Design and Simulation of a Secure Enterprise Network Using VLANs, Inter-VLAN Routing, DHCP, NAT/PAT, ACLs and Secure Device Management

| Name: | Matchoffo Lepe Miran |
| --- | --- |
| Track: | Computer Networking and Cybersecurity |

1. Introduction

This project involved the design and simulation of a secure enterprise network using Cisco Packet Tracer. The network was designed to connect different departments while providing proper network segmentation, communication, automatic IP addressing, and security.

2. Problem Statement

Organizations require secure and well-organized networks to separate different departments and prevent unauthorized access. Without proper network segmentation and security, network traffic can become difficult to manage and sensitive resources may be exposed.

3. Objectives

The objectives of the project were to:

- Design an enterprise network topology.

- Separate departments using VLANs.

- Configure Inter-VLAN Routing.

- Configure DHCP for automatic IP addressing.

- Implement NAT/PAT.

- Configure ACLs to control network traffic.

- Implement secure device management using SSH.

- Test and verify network connectivity and security.

4. Network Design

The network consisted of:

- 2 Routers

- 4 Switches

- 10 PCs

- 1 Server

The network was divided into different departments using VLANs.

| VLAN | Department | Network |
| --- | --- | --- |
| VLAN 10 | Administration | 192.168.10.0/24 |
| VLAN 20 | Finance | 192.168.20.0/24 |
| VLAN 30 | IT | 192.168.30.0/24 |
| VLAN 40 | Sales | 192.168.40.0/24 |
| VLAN 50 | Server | 192.168.50.0/24 |
| VLAN 60 | Guest | 192.168.60.0/24 |
| VLAN 99 | Management | 192.168.99.0/24 |

5. Technologies Implemented

VLANs

VLANs were used to separate the different departments, improve security, and reduce unnecessary broadcast traffic.

Inter-VLAN Routing

Inter-VLAN Routing was configured to allow controlled communication between devices in different VLANs.

DHCP

DHCP was configured to automatically assign IP addresses, subnet masks, default gateways, and DNS information to client devices.

NAT/PAT

NAT/PAT was implemented to translate private IP addresses and allow multiple internal devices to share an external IP address.

ACLs

ACLs were configured to control network traffic and restrict unauthorized access. The Guest network was prevented from accessing protected internal networks.

Secure Device Management

SSH, encrypted passwords, enable secrets, usernames, and management VLANs were configured to provide secure access to network devices.

Port Security

Port security was implemented to help prevent unauthorized devices from connecting to switch ports.

6. Testing and Results

The network was tested using ping and verification commands.

The following tests were performed:

- VLAN configuration testing

- Trunk link testing

- DHCP address assignment

- Inter-VLAN connectivity testing

- ACL restriction testing

- NAT/PAT verification

- SSH connectivity testing

The results showed that the network was successfully segmented, devices received IP addresses automatically, authorized VLAN communication was possible, and unauthorized access was restricted.

- Challenges and Solutions

| Challenge | Solution |
| --- | --- |
| Communication between VLANs | Configured Inter-VLAN Routing |
| IP address management | Configured DHCP |
| VLAN traffic between switches | Configured trunk links |
| Secure remote management | Configured SSH |

8. Conclusion

The project successfully demonstrated the design and simulation of a secure enterprise network using Cisco Packet Tracer. VLANs were used to separate departments, while Inter-VLAN Routing enabled controlled communication. DHCP simplified IP address management, NAT/PAT supported address translation, and ACLs improved network security. SSH and other security features provided secure device management.

Overall, the project achieved its objective of creating a secure, organized, and manageable enterprise network.